Code Manual

Click the icon in the red box to open a selection of all materials

Click on the Senior-Project-Code if current material selected isn’t the material you need.

Select required material, available are Concrete, Wood, Masonry

To Run either Wood or Masonry click the run button. Note: For Masonry there is no UI, go through the top half where the notes say to edit and change the input values.

Then answer pop up questions.

After printout in Console copy paste code into any word doc of your choosing, ensure that you also take any plots with it.

Variables

Concrete Beam Varriables:

aggergate size: Size of aggeragate used in concrete in (in.)

b: The width of the beam being analyzed in (in.)

bands: The different groupings of the stirrups.

bending: For non rectangular sections is the beam bending positively or negativley.

cover: The amount of concrete covering any rebar from the air in (in.)

d: The height from the top of the compression face to the mid depth of the rebar in (in.)

d1: The height from the top of the compression face to the mid depth of the first layer of rebar in (in.)

d2: The height from the top of the compression face to the mid depth of the second layer of rebar in (in.)

f’c: Compressive yield strength of the concrete in (psi.)

fy: Yeild strength of the flexural steel in (ksi)

fy: Yeild strength of the shear steel in (ksi)

h: The height of the beam being analyzed in (in.)

length total: The full length of the beam in (ft.)

live load percent sustained: The percent of live load applicable after the concrete is dry in (%.)

Nu: Any outside compressive force on the beam in (k.)

number of rebar: The number of rebar bars in the beam.

rebar size: The size of the rebar in the beam provided range is 3-11.

spacing: The distance between the rebar in (in.)

stirrup leggs: The number of leggs a stirrup has.

time dependent varriable: per ACI18-19(22) table 24.2.4.1.3.

weight of concete: The cubic weight of the concrete in (pcf.)

Vu: The maximum shear in the beam in (k.)

CMU Varriables:

Bar count horizontal: The number of bars going along the beam.

Bar size horizontal: The size of the bar going along the beam.

Bar size vertical: The size of the bar going vertically in the beam.

Beam Courses: The number of CMU bricks stacked that are in the beam.

CMU Size: The block size in the form _x_x_ with _ in (in.)

Dead load: Any perminate load that doesn’t include self weight. Units as asked for.

Es: Modulus of Elasticity. SHOULDN’T NEED TO CHANGE.

Floor depth: How far the beam supports the wall. The trib depth. Units (ft.)

fm = compressive strength of the CMU in (ksi.)

fy: Yeild strength of the steel in (ksi.)

gap: The distance you want between the rebar and the face of the CMU in (in.)

Grout fill: The patern in which grout is placed in the CMU.

Length: The length of the beams span in (ft.) DON’T include bearing.

Length Bearing: The length of bearing of the beam in (in.)

Mortar type: Type of mortar used to bond the CMU. Options are M, S, N.

Number of Courses: The number of bricks above the beam, doesn’t include what is in the beam.

Point load: Must be factored and may lead to error since it is not implemented for arching action.

Self load: Any load from occupancy. Units as aked for.

Space: The gap between the vertical bars in (in.)

Support: What is the beams support condition. Either Cantilever or Simply Supported.